# When to choose a soft starter over a variable frequency drive (VFD)

In industrial settings, choosing between a soft starter and a variable frequency drive (VFD) can be pivotal in optimising motor performance, efficiency and cost-effectiveness. Both technologies serve the purpose of controlling motors, but their application, benefits and limitations vary significantly. Here Ali Khalil, development sales manager for high voltage solutions at industrial equipment manufacturer WEG explains that deciding when to use a soft starter instead of a VFD involves assessing four key factors: speed control, price, size and energy efficiency.

The motor starter market is poised to reach USD 13 billion by the end of 2026, growing at a CAGR of five per cent during this period. While VFDs are versatile and suitable for almost all applications, soft starters are more specific in their use cases. This doesn't mean that VFDs are easier to specify, install and commission — it simply means they are applicable to a wider variety of scenarios. Let’s explore when you should choose a soft starter over a VFD.

When smooth starts are enough

The primary distinction between a soft starter and a VFD is to the way they control speed. A soft starter gradually ramps up the voltage to allow the motor to start smoothly, minimising torque and mechanical stress during the startup phase. However, once the motor reaches full speed, a soft starter cannot provide any additional speed control. This makes soft starters ideal for applications in conveyors, pumps and fans, where motors only need to operate at full speed and the primary concern is reducing inrush current and mechanical stress during startup.

Therefore, when an application requires consistent speed adjustments and torque control throughout the motor's operation, a VFD is the clear choice. However, for applications that do not need speed control beyond startup, a soft starter is a more efficient option.

Weighing the cost of simplicity versus advanced control

Cost is another critical factor when deciding between a soft starter and a VFD. Soft starters are generally more affordable due to their simpler design and functionality. A soft starter uses silicon-controlled rectifiers (SCRs) to connect the AC input line to the motor. In contrast, VFDs incorporate more complex components including a diode front end, DC bus and insulated-gate bipolar transistors (IGBTs), which increase costs.

This additional complexity makes VFDs more expensive, often two to three times the price of a soft starter, especially for larger horsepower applications. However, the higher initial cost of a VFD can sometimes be justified by its potential to deliver long-term energy savings, particularly in applications where motors frequently run at partial loads or variable speeds.

Maximising space with compact solutions

The next consideration should be size. Soft starters also offer substantial savings in physical size and volume compared to VFDs. The additional components in a VFD and its cooling system not only cost more but also take up more space. The volume savings with a soft starter depend on the power rating. In industries where space is limited, such as shipyards or small manufacturing facilities, the smaller footprint of a soft starter can be a crucial advantage. The reduced size and complexity also mean that soft starters are easier and quicker to install, further reducing operational costs.

Instant efficiency gains or long-term savings with speed control

Finally, energy efficiency should be considered. Soft starters can achieve up to 99.5 per cent to 99.9 per cent efficiency. After the starting process, a soft starter with an integrated bypass activates an internal contactor, allowing current to flow through it efficiently. In contrast, VFDs are around 95 per cent efficient because active components like IGBTs are always on during start, run and stop phases, leading to lower efficiency. However, VFDs offer more opportunities for energy savings by controlling both voltage and frequency.

When operating at full speed and properly loaded, soft starters are more efficient than VFDs. Once up to speed and bypassed, the motor current flows across the bypass contactor in a soft starter, with no active solid-state components generating heat. Conversely, a VFD remains hotter due to its active components constantly managing voltage and frequency.

It's clear that properly selected soft starters can yield significant cost, size and energy savings over VFDs. The latter can be two to seven times more expensive and two to eight times larger than soft starters, but application characteristics are critical when choosing between the two options.

Both can start a motor with reduced voltage and current, but the unique aspects of your application will determine the best choice, allowing you to right-size either a soft starter or a drive. Regardless of the motor control method you choose, consider additional factors such as installation, commissioning and the need for extra filters or circuit protection when specifying a soft starter or a VFD.

Visit WEG’s website to learn more about the company’s range of soft starters and VFDs.

Ends: 803 words

Image caption:

Editor’s note: If you want to ensure you keep up to date with press material, opinion focussed content and case studies from WEG visit https://www.weg.net/institutional/MR/en/all-news/all

For further information contact: Marek Lukaszczyk

Broad Ground Road, Lakeside, Redditch, Worcestershire B98 8YP

www: www.weg.net

e-mail: info-uk@weg.net

Press enquiries: Laura England or Stephanie Wood – Stone Junction Ltd

Suites 1&2 The Malthouse, Water Street, Stafford, Staffordshire, ST16 1AR 
Telephone: +44 (0) 1785 225416

e-mail: laura@stonejunction.co.uk or stephanie@stonejunction.co.uk

www: www.stonejunction.co.uk

Blog: www.stone-junction.blogspot.com

Twitter: www.twitter.com/StoneJunctionPR

Facebook: http://www.facebook.com/technicalPR

LinkedIn: https://www.linkedin.com/company/stone-junction-ltd

About WEG: Founded in 1961, WEG is a global electric-electronic equipment company, operating mainly in the capital goods sector with solutions in electric machines, automation and paints for several sectors, including infrastructure, steel, pulp and paper, oil and gas, mining, among many others.

WEG stands out in innovation by constantly developing solutions to meet the major trends in energy efficiency, renewable energy and electric mobility. With manufacturing units in 15 countries and present in more than 120 countries, the company has more than 40,000 employees worldwide. WEG’s net revenue reached R$ 32.5 billion in 2023, 53 per cent from external markets. For more information, visit www.weg.net

Ref: WEG1183/09/24